IADK Version 2 User Guide

Developing for Red Lion IPm RTUs

CONFIDENTIAL AND PROPRIETARY

The information in this document is proprietary to Red Lion Controls.

The information shall not be used, copied, reproduced or disclosed in whole or in part without written consent of Red Lion Controls.

Copyright 2014 Red Lion Controls.

All rights reserved.

IADK Version 2 User Guide

Developing for Red Lion IPm RTUs

Table of Contents

Overview	4

Requirements	4

Installation	5

Linux Desktop	5

Virtual Machine	5

Logging In	6

Building the Sample Program	6

IADK Libraries	6

IODB	6

LibUDR	6

Using Serial Ports	6

Remote Debugging	7

Building for Debugging	7

Configuring gdbserver	7

Running gdb	8

Appendix 1 – Shared Directories	9

Appendix 2 – License Notice	12

Software License Notice	12

Software License Agreement	12

IPm OEM Licensing Agreement	12

General Public License	12

Table of Figures

Figure 1 - Shared Folders in Virtual Box Settings	9

Figure 2 - Batch File to Map Shared Directory	10

Figure 3 - fstab Entry for Shared Directories	10

Figure 4 - rc.local Script to Automatically mount Shared Directory	10

IADK Version 2 User Guide

Developing for Red Lion IPm RTUs

# Overview

Red Lion’s IPm line of Linux-based Remote Terminal Units (RTUs) is user programmable to accommodate many diverse applications. The ISaGRAF subsystem may be used to create programs in a subset of several IEC 61131 languages. ISaGRAF programs use standard and IPm-specific function blocks and are extremely portable.

Users are encouraged to use ISaGRAF for programming whenever possible. However, for some applications, portability must be sacrificed for greater power and flexibility. The IPm Application Development Kit allows users to write C programs which interact with the Linux operating system, RTU hardware, and other automation applications.

Version 1 of the IADK (IADKv1) included custom-built, Windows- and Linux-based tools for creating automation applications in C. IADKv1 supports all RTUs based on the PowerPC 855T SoC.

In 2014, Red Lion released a new set of IPm RTUs based on the more powerful 8313 SoC and version 2 of the IADK (IADKv2) for users to create C programs for them. IADKv2 is based on more modern and standard tools and includes more libraries to ease application development. In many cases, programs created with IADKv1 can be recompiled with version 2 with few changes. In some cases, the more pedantic modern compiler in IADKv2 warns about questionable program constructs which may be latent defects in existing code. In many cases the revised application source can be recompiled with version 1 for existing RTUs and version 2 for the new RTUs.

This document describes the features and use of IADKv2. Information on IADKv1 is available at sixnet.com. Information on ISaGRAF programming is distributed with the ISaGRAF product.

# Requirements

IADKv2 is based on the Embedded Linux Development Kit (ELDK) from Denx. The tools in the ELDK run on a “Lintel” x86-based desktop Linux system and target various PPC- and ARM-based embedded systems. Therefore, developing for 8313-based IPm RTUs requires:

- A desktop Linux computer. Red Lion recommends Ubuntu 14.04 but other distributions compatible with the ELDK may work.

- ELDK 5.3

- Libraries, documentation, and sample programs from Red Lion. These are provided with the purchase of the IPm Advanced License in the Sixnet series I/O Tool Kit.

Note: Use of the IPm Application Development Kit requires the purchase of the IPm Advanced License (option group 4) of the SIXNET I/O Tool Kit.

For those without a Linux computer (or with an incompatible one), Red Lion provides a virtual machine (VM) with Ubuntu 14.04, generic Linux development tools (editors, etc.), the ELDK, and Red Lion libraries and samples preinstalled. The VM is tested to run in Virtual Box 4.2 and later. Other virtual machine programs like VMWare may also work.

# Installation

## Linux Desktop

- Get the IADK by cloning the IADK repository

$ git clone http://source.sixnet.com/git/iadkv2.git

- Install ELDK using the script provided with the IADK

$ cd iadkv2

$ ./bin/get-eldk5.3

## Virtual Machine

The IADKv2 virtual machine may be run on any operating system that supports suitable host software. VirtualBox runs on Windows, Max OS X, Linux, and Solaris.

- Obtain the Open Virtual Appliance file from Red Lion. It may be downloaded from RedLion.net. If you prefer to avoid the 5GB download, contact Red Lion support to get the appliance on a USB drive.

- Install suitable Virtual Machine host software such as VirtualBox.

- Import the IADKv2 VM into your host software. With VirtualBox, follow these steps:

  - Run VirtualBox Manager

  - Pick Import Appliance… from the File menu

  - In the Import Virtual Appliance wizard, click Open appliance… and navigate to CPU-D IADK 20141010.ova and select it then click Next in the wizard

  - Adjust the Appliance Settings as desired. Be sure to check Reinitialize the MAC address of all network cards

  - Click Import. (The import process can take some time to complete.)

- Configure Shared Directories, if desired. (See Appendix 1 – Shared Directories.)

### Logging In

The IADK VM has a user named iadk with a default password cpud. You may wish to change this password or remove the user and add your own using standard Linux administration tools.

Once logged in, you should update the libraries distributed in the VM to the latest version.

iadk@iadk-VirtualBox:~$ cd iadkv2/

iadk@iadk-VirtualBox:~/iadkv2$ git pull

Already up-to-date.

iadk@iadk-VirtualBox:~/iadkv2$ cd ..

iadk@iadk-VirtualBox:~$ cd sxlibs/

iadk@iadk-VirtualBox:~/sxlibs$ git pull

Already up-to-date.

iadk@iadk-VirtualBox:~/sxlibs$

# Building the Sample Program

The IADK comes with a sample program, oem_iodb, which shows how to access the I/O Database.

The make file in the iadkv2 directory builds both of these programs. Once built you can copy them to your IPm with ftp and test them there.

# IADK Libraries

## IODB

The IODB library provides functions to read and write values in the I/O Database, including writing physical outputs. Documentation is available online at sixnet.com.

## LibUDR

LibUDR provides functions to send and receive I/O values between stations or your PC, including using authentication to limit those operations. Documentation for LibUDR can be viewed by opening sxlibs/docs/libudr/html/index.html in a web browser.

# Using Serial Ports

You can access the serial ports on an IPm station in the same manner as standard Linux, using the open() system call.

The ST-IPM-8460, VT-IPM-138-D, and VT-IPM-248-D serial devices are as follows:

| Model | Port A | Port B | Port C | Port D |
| --- | --- | --- | --- | --- |
| ST-IPM-8460 | ttyS2 | ttyS0 | ttyS3 | ttyS1 |
| VT-IPM-138-D | ttyS2 | ttyS0 |  | ttyS1 |
| VT-IPM-248-D | ttyS2 | ttyS0 | ttyS3 | ttyS1 |

In order to use a serial port for a custom application, the port mode must be set to User or Unassigned in the SIXNET I/O Tool Kit Serial Port Configuration window.

- User mode means the Tool Kit sets the port parameters such as baud rate, parity, etc.

- Unassigned mode means it is your application's responsibility to set up the port.

# Remote Debugging

Programs developed with the IADK may be debugged interactively via gdb, the GNU debugger. The debugger runs on your development system (VM or Linux desktop) and communicates with gdbserver running on the RTU. gdb is a complex, command-line program. The GDB Wiki at Sourceware.org lists numerous GUI options which may simplify its use. General gdb use and GUI applications are beyond the scope of this document. The following paragraphs describe running gdb and gdbserver.

## Building for Debugging

To enable debugging, your program must be built with the –ggdb option. Your command line or make file should include something like

$ powerpc-linux-gcc -ggdb -o hello hello.c

## Configuring gdbserver

gdbserver runs on the RTU and acts as a proxy for gdb on the development system to control and monitor the program being debugged. The two communicate via serial port or Ethernet network. The general form of the gdbserver command is

gdbserver comm prog args

Where

- comm is a serial device like /dev/ttys0 or an IP address and port like 10.1.0.1:2345

- prog is the program being debugged. The name must include the a full or relative path if the program is not in a directory on your $PATH.

- args are arguments to the program being debugged

For example, to debug hello in the current directory via Ethernet you might do

$ gdbserver 10.1.0.1:2345 ./hello

Alternatively, an already-running program may be adopted by gdbserver with a command like:

$ gdbserver comm –attach pid

Where

- pid is the process ID of the running program

## Running gdb

gdb needs local access to a copy of the program you are doing to debug in the RTU. Likely, this means building it (with –ggdb) then copying the output to the RTU and using the original on your development system. On the development system, you start gdb and pass the name of the program to be debugged like:

$ powerpc-gdb hello

Then at the gdb prompt, tell it that the target is the program on your remote device. For serial this is something like:

(gdb) target remote /dev/ttyb

Where /dev/ttyb is the name of the serial port on the development system.

For network-based debugging, you would enter something like:

(gdb) target remote 10.1.0.1:2345

Where the IP address and port number match what you told gdbserver on the RTU.

# Appendix 1 – Shared Directories

Using shared directories (or folders) files can be made visible to both the Linux VM and the host operating system. For example, when using Windows as a host operating system, you can make V: in Windows represent /v in the Linux VM or My Documents/Source represent ~/src.

You can map V: in Windows to /v in the Linux VM using VirtualBox, as follows:

- Create a Shared Folder in the VM Settings

  - Run VirtualBox Manager

  - Select the VM you want to configure

  - Click Settings

  - Select Shared Folders

  - Click the  icon to add a folder (See Error: Reference source not found)

Figure 1 - Shared Folders in Virtual Box Settings

  - In the Add Share dialog pick Other… from the Folder Path dropdown and browse to the directory you want to share.

    - Change the Folder Name if you wish. (This name will be used when setting up fstab in step 3, below.)

    - Make sure neither Read-only nor Auto-mount is checked.

  - Click OK to close the Settings dialog

- Create a virtual drive for the shared directory (Optional)

  - Create a batch file to map the drive

    - Run Notepad or another text editor and enter the text like that in Error: Reference source not found using the drive letter of your choice and the Folder Path used to create the VirtualBox shared folder, above.

    - Save the file with a .bat extension on your desktop (e.g., map_shared.bat)

# VirtualBox shared folder

SUBST V: C:\Users\someuser\V

Figure 2 - Batch File to Map Shared Directory

  - Open the Startup programs folder

    - Run Windows Explorer (e.g., by clicking Start and choosing Computer)

    - Navigate to C:\ProgramData\Microsoft\Windows\Start Menu\Programs\Startup\

  - Drag the batch file created above to the Startup folder

  - Double click on the file to run it and establish the new virtual drive for the first time. The next time you log in, you should find the mapping already in place

- Tell Linux where to mount the shared directory

  - Edit /etc/fstab and add text like that in Error: Reference source not found. The first thing on the last line is the Folder Name of the shared folder from VirtualBox settings, above.

# Shared folder for copying data between host and Linux VM

# Mounted in /etc/rc.local

V /v vboxsf defaults,noauto 0 0

Figure 3 - fstab Entry for Shared Directories

  - Create the mount point with a command like that in Figure 4. The directory you create must match the second item in the line you added to /etc/fstab, above.

$ sudo mkdir /v

Figure 4 - Command to Create Mount Point

- Tell Linux to mount the shared directory when it boots

  - Edit /etc/rc.local and change the end to look like Figure 5. The argument to mount (/v in the example) must match the second item in the line you added to fstab.

# Mount VirtualBox shared folder

mount /v

exit 0

Figure 5 - rc.local Script to Automatically mount Shared Directory

# Appendix 2 – License Notice

## Software License Notice

The software supplied by Red Lion Controls, including the software development tools we supply and the firmware that executes within our hardware products, is a combination of Red Lion proprietary software, general public license software, and other third party software. Below are listed three software licenses that govern the applicable software components and products that we supply. In addition to these three license agreements, third party licenses may apply to specific software products supplied by others and must be complied with in accordance with their terms and conditions. Copies of these licenses may be referenced in a help topic that is accessed through the Help menu in the I/O Tool Kit software.

## Software License Agreement

This is the license agreement that you must accept in order to register any SIXNET development tools, including the I/O Tool Kit software.

## IPm OEM Licensing Agreement

This agreement gives OEMs the right to use or modify Red Lion products. You must accept the terms of this agreement in order to register features in the I/O Tool Kit software that grant you license to modify software in an IPm controller or if you plan to private label any Red Lion products.

## General Public License

This license applies to software included in IPm controllers and other applicable products that contain software supplied as Public Licensed Software. Red Lion publishes this license as a requirement of incorporating GPL (General Public Licensed) software into our products and to encourage you to do the same with applicable software that you write.

This document applies to IPm models ST-IPM-8460, VT-MIPM-138, VT-MIPM-248.

ST-IPM-8460, VT-IPM-138-D, and VT-IPM-248-D models.

The IADK VM has been tested with VirtualBox 4.2.18. Later versions likely work but have not been tested.

The appliance file name ends with the date it was created in yyyymmdd format. Use the latest available.